# High-Level Design Document: Robotic Animation Controller

System Requirements, Design Tradeoffs, and Architectural Overview

# 1. Introduction

This document outlines the high-level design for a Robotic Animation Controller (RAC), a system intended to synchronize and control multiple robotic actuators for complex, coordinated movements. The RAC is envisioned for use in variety of themed entertainment installations such as parade floats, animated displays, holiday decorations, etc..  The initial driver for this system was during the 2025 la Canada Flintridge Tournament of Roses parade float development.  During this year there was a transition from conventional hydraulic actuators to electronic animation.  A need arose for convenient means to plan, program and execute animated motions using motors, servos and valves which were new to the float development teams.

# 2. Key Requirements

- Must be able to control a series of motion actuators in real-time using a programmable animation sequence.

- All animation sequences shall be synchronized with all other controlled motion actuators.

- Scalability: The system should support a variable number of actuators, from one to a minimum of 16.

- Configurability: Users must be able to define and modify animation sequences easily, through a scripted sequence of motion steps.

- Human Machine Interface: The controller must provide physical inputs to manage the selection, starting, stopping, and pausing the motion sequences.

- Interoperability: The controller should communicate with various actuator types (servo motors, linear actuators, pneumatic devices) via standard protocols (e.g., RS-422, RS-232, etc.).

- Safety: Built-in safety mechanisms must prevent actuator collisions and excessive stress, with emergency stop and manual override functions.

# 3. Design Tradeoffs

- Centralized vs. Distributed Control:

- Centralized control simplifies synchronization but may become a bottleneck as actuator count increases.

- Distributed control improves scalability and fault tolerance but complicates synchronization and increases network traffic.

Real-Time OS vs. Standard OS:

Real-time operating systems offer deterministic timing, essential for precise animation, but may limit software and hardware compatibility.

Standard OSes provide greater flexibility and broader driver support but may introduce unpredictable latencies.

Wired vs. Wireless Communication:

Wired protocols (CAN, RS-485) offer reliable, low-latency communication ideal for synchronized control.

Wireless options (Wi-Fi, Bluetooth) increase flexibility but are susceptible to interference and latency, generally not recommended for core synchronization.

Custom vs. Off-the-Shelf Hardware:

Custom solutions can be optimized for performance and size but increase development time and cost.

Off-the-shelf hardware accelerates development and reduces risk but may limit the controller’s capabilities.

# 3. Proposed System Design

12VDC Batteries + Converters

Actuator Assembly #1

Motor Controller

Motor

Pos Encoder

Limit Switches

Animation Controller

CPU

User Interface

Animations

Com Links

Com Links

Com Links

Com Links

Actuator Assembly #2

Motor Controller

Motor

Pos Encoder

Limit Switches

Actuator Assembly #16

Motor Controller

Motor

Pos Encoder

Limit Switches

12VDC

12VDC

12VDC

RS422

# 4. System Architecture Overview

## 4.1. Component Diagram

- User Interface Module: Provides tools for animation creation, actuator configuration, and real-time monitoring.

- Sequencer Engine: Manages animation timelines, synchronizes actuator commands, and resolves timing conflicts.

- Actuator Abstraction Layer: Translates high-level animation commands into protocol-specific instructions for each actuator type.

- Communication Manager: Handles data transmission to and from actuators via selected protocols, supporting both polling and event-driven models.

- Fault Detection & Safety Module: Monitors actuator health, enforces safe operation limits, and initiates emergency procedures if necessary.

- Logging & Analytics Module: Records system events, actuator performance, and user actions for diagnostics and optimization.

## 4.2. Data Flow

- User creates or loads an animation sequence via the User Interface.

- Sequencer Engine schedules actuator actions according to the timeline and synchronizes commands.

- Actuator Abstraction Layer maps generic commands to specific actuator protocols and parameters.

- Communication Manager delivers instructions to actuators and receives feedback.

- Fault Detection & Safety Module continually monitors system status and intervenes as required.

- Logging & Analytics Module stores relevant data for review and optimization.

## 4.3. Technology Stack Recommendations

- Programming Language: C++ or Python for core modules; JavaScript for GUI.

- Operating System: Linux with real-time kernel extensions or a dedicated RTOS.

- Communication Protocols: CAN bus, RS-485, Ethernet (TCP/UDP).

- Hardware: Industrial-grade microcontrollers or embedded PCs, depending on scalability needs.

# 5. Conclusion

The Robotic Animation Controller is designed to offer robust, scalable, and precise control over a network of actuators, enabling synchronized robotic performances. By considering key requirements and evaluating critical tradeoffs, this architecture provides a flexible foundation for both current and future animation control needs. Further detailed design and prototyping will refine module implementations and validate system performance in real-world scenarios.